MASTERHUB BCI CORTEX INTEGRATION System Architecture, Signal Fluctuation Analysis & Failure Diagnostic Report Target: Emotiv EPOC X Headset ¦ Interface: Cortex v2 API (WSS JSON-RPC 2.0) ¦ Host: MasterHub Engine

1. Executive Summary

MasterHub incorporates a real-time Brain-Computer Interface (BCI) subsystem designed to connect with an Emotiv EPOC X headset via the Emotiv Cortex v2 API. The subsystem receives live mental commands, cognitive metrics, and contact-quality diagnostics to drive multimodal actuation across Robotics, IoT Smart Home, and Assisted Living domains.

Live diagnostic monitoring reveals that the Cortex connection is currently stalled in a continuous reconnection loop. While the underlying Secure WebSocket transport (wss://localhost:6868) handshakes successfully, authentication is halted at the authorization stage because the application access has not been approved in the EMOTIV Launcher and the environment configuration contains placeholder credentials. This document details the end-to-end architecture, analyzes the sources of physical and protocol connection fluctuations, presents the empirical failure trace, and provides the step-by-step remediation plan.

[SYSTEM STATUS SUMMARY] Current Status: WebSocket transport active; Authorization BLOCKED due to unapproved Client ID and placeholder secret in .env. Live BCI telemetry ingestion is suspended until authorization is completed.

2. End-to-End Cortex Connection Architecture

The connection between the Emotiv EPOC X headset and MasterHub spans five distinct layers, bridging biological neural signals to robotic and IoT domain actuators:

| Layer | Module / Component | Protocol / Physical Medium | Functional Responsibility |
| --- | --- | --- | --- |
| 1. Physical / Scalp | Emotiv EPOC X Electrodes | 14 Saline Felt Pads + 2 Mastoids | Microvolt EEG capture, 128/256Hz ADC sampling |
| 2. Wireless Transport | 2.4 GHz RF / BLE Receiver | Proprietary RF / Bluetooth USB Dongle | Wireless packet transmission from headset to Host PC |
| 3. Emotiv Daemon | EMOTIV Launcher / Cortex Service | wss://localhost:6868 (JSON-RPC 2.0) | Driver layer, signal decryption, FFT & ML classification |
| 4. MasterHub Client | cortex/cortex_client.py & auth.py | Secure WebSocket Client (WSS) | RPC calls, ping/pong heartbeats, token refresh, sessions |
| 5. Stream & Gating | cortex/stream.py & control.py | Thread Queue & Normalized Schema | Ingests 'com' stream, neutral-release gating, FSM dispatch |
| 6. Core Execution | core/engine.py & core/state.py | Internal MasterHub Router | Translates gestures (Push, Pull, Left, Right) to domain actions |

2.1 Detailed Handshake & Connection Lifecycle

The connection is orchestrated by the LiveRunner worker thread (cortex/run_live.py) following a strict 7-phase sequence:

1. WebSocket Connection: Establishes a TLS-encrypted WebSocket to wss://localhost:6868 with VERIFY_SSL=False (accommodating Emotiv's local self-signed certificate).

2. Access Verification (requestAccess): Verifies that MasterHub's Client ID has been granted access by the user in the EMOTIV Launcher.

3. Authorization (authorize): Exchanges the Client ID, Client Secret, and license key for a temporary bearer cortexToken (with a 6-hour soft TTL auto-refresh).

4. Headset Discovery & Rescan: Queries visible headsets via queryHeadsets; executes controlDevice(command='refresh') if no headset is initially detected.

5. Headset Pairing: Issues controlDevice(command='connect', headset=id) to establish the active RF bond.

6. Session Creation & Stale Eviction: Opens an active session (createSession). If a previous crashed session locked the device (-32002 error), MasterHub inspects querySessions and evicts stale locks automatically.

7. Telemetry Subscription: Subscribes to the Mental Command stream (subscribe(streams=['com'])). Incoming packets at 10-20 Hz are converted into NormalizedPrediction objects.

3. Complete Signal Fluctuation & Instability Analysis

Users of BCI headsets frequently experience intermittent signal drops or delayed response times. These fluctuations originate across five distinct physical and algorithmic boundaries:

| Instability Tier | Root Mechanism | System Impact | Engineering Mitigation in MasterHub |
| --- | --- | --- | --- |
| Tier 1: Electrode Contact | Saline felt pads drying out or poor scalp contact (>10 kOhm impedance) | Signal quality drops; Cortex ML defaults to 'neutral' or low confidence | Periodic 'dev' diagnostics monitoring; signal quality warnings on dashboard |
| Tier 2: Wireless RF Noise | USB 3.0 unshielded port radiation & 2.4GHz Wi-Fi congestion | Headset state flips between 'connected' and 'discovered'; packet loss | Use USB 2.0 port or extension cable; automatic connection pooling with 4s rescan |
| Tier 3: Daemon Lockouts | Ungraceful disconnect leaves session locked in Cortex daemon for 60s | createSession rejected with error -32002 ('headset already in use') | Automatic stale session query and eviction implemented in cortex/session.py |
| Tier 4: Token Expiration | Cortex tokens silently expire without machine-readable timestamps | RPC calls suddenly fail with authentication errors | CortexAuth proactive 6-hour soft-TTL refresh and auto-recovery thread |
| Tier 5: Algorithmic Gating | Rapid back-to-back mental commands without relaxation | Commands dropped by stabilizer to prevent accidental runaway execution | Neutral-Release Gating (>=0.25s neutral) and Command Hold (>=0.35s sustained hold) |

[OPERATIONAL GATING REQUIREMENT] Crucial Operational Rule: BCI mental control requires the user to return to 'neutral' (relaxation) for at least 0.25 seconds between consecutive actions. Holding a continuous 'Push' will intentionally not trigger repetitive selections in MasterHub's safety state machine.

4. Live Connection Failure Incident Report

Empirical examination of the live runtime logs (cortex/logs/cortex.log) reveals that the system is currently failing at the authentication stage. Below is the exact diagnostic trace extracted from the active server:

2026-09-24 09:55:58 ¦ INFO ¦ cortex.cortex_client ¦ Connected to Cortex at wss://localhost:6868 2026-09-24 09:55:58 ¦ DEBUG ¦ cortex.cortex_client ¦ -> requestAccess (id=438) params={'clientId': '4peZxtpflilSEnDpunLvBCOImDBQOnsFkFeJzgZS', 'clientSecret': 'TEST_SECRET_KEY_XYZ_456'} 2026-09-24 09:55:58 ¦ DEBUG ¦ cortex.cortex_client ¦ <- requestAccess (id=438) OK 2026-09-24 09:55:58 ¦ WARNING ¦ cortex.auth ¦ Cortex access NOT yet granted: The user has not granted access right to this application. Please use EMOTIV Launcher to proceed. 2026-09-24 09:55:58 ¦ WARNING ¦ cortex.run_live ¦ Cortex unavailable: Cortex access has not been granted for this CLIENT_ID. Open the EMOTIV Launcher/App, approve the access request popup for this application, then retry. 2026-09-24 09:55:58 ¦ INFO ¦ cortex.cortex_client ¦ Cortex client closed

4.1 Detailed Breakdown of Failure Causes

1. Unapproved Application in EMOTIV Launcher: The requestAccess JSON-RPC call returns accessGranted: false. The Emotiv Cortex security model mandates that the desktop user must explicitly click 'Approve' on the application authorization popup inside the EMOTIV Launcher GUI.

2. Invalid / Placeholder Client Secret in .env: Inspection of the .env file shows CORTEX_CLIENT_SECRET=TEST_SECRET_KEY_XYZ_456. This dummy value prevents successful cryptographic token generation (authorize method) once access is requested.

3. Fail-Fast Socket Tear-Down: When LiveRunner encounters CortexAuthError, it closes the socket and triggers a 5-second backoff. Because the approval remains unhandled in the Launcher, the connection enters an infinite connect -> requestAccess -> reject -> close loop.

5. Step-by-Step Remediation & Setup Guide

To resolve the failure and achieve stable, live BCI telemetry streaming, follow these operational procedures in order:

• Step 1: Retrieve Emotiv Developer Credentials

Navigate to https://www.emotiv.com/my-account/cortex-apps/ in your browser. Create or select your application to copy your official Client ID and Client Secret.

• Step 2: Update the Environment Configuration (.env)

Open the .env file in the MasterHub root directory and replace the placeholder values with your production credentials:

CORTEX_CLIENT_ID=your_real_emotiv_client_id CORTEX_CLIENT_SECRET=your_real_emotiv_client_secret CORTEX_URL=wss://localhost:6868 CORTEX_VERIFY_SSL=false CORTEX_STREAM_NAME=com CORTEX_SESSION_STATUS=active BCI_CONFIDENCE_THRESHOLD=0.40

• Step 3: Authorize MasterHub in EMOTIV Launcher

Launch the EMOTIV Launcher desktop application. Start MasterHub (python app.py). An access-request prompt will appear in the Launcher stating 'Application [App Name] is requesting access'. Click Approve.

• Step 4: Prepare and Hydrate the EPOC X Headset

Ensure all 14 felt pads are saturated with saline solution. Fit the headset to ensure reference sensors (CMS/DRL) have solid contact against the mastoid bones. Verify green contact status in the EMOTIV Launcher.

• Step 5: Verify Live Ingestion via MasterHub Dashboard

Open your web browser to http://127.0.0.1:5000/cortex/ or query http://127.0.0.1:5000/cortex/state to verify that status.connected=true, status.authorized=true, and live gestures are updating in real time.

6. Data Schema & Telemetry Reference

The table below defines the mapping between raw Cortex JSON-RPC payloads and MasterHub's normalized internal telemetry schema:

| Field | Raw Cortex Payload | MasterHub Normalized Schema | Description / Value Range |
| --- | --- | --- | --- |
| Mental Command | msg['com'][0] | prediction.command / gesture | Cognitive action: 'neutral', 'push', 'pull', 'left', 'right' |
| Confidence Score | msg['com'][1] | prediction.confidence / power | Normalized floating-point probability: 0.00 to 1.00 |
| Timestamp | msg['time'] (Unix float) | prediction.timestamp (ISO 8601) | Universal UTC timestamp with microsecond resolution |
| Session ID | msg['sid'] (UUID string) | prediction.session_id | Unique session identifier assigned by Cortex daemon |
| Battery Level | msg['dev'][1] (Integer %) | headset.battery_percent | Headset battery percentage: 0% to 100% |
| Signal Quality | msg['dev'][3] (Float) | headset.signal_quality | Composite contact quality metric: 0.0 to 1.0 (Good > 0.8) |